Wireless Enterprise Standard

# Status: Draft

Wireless Day0, 1 and 2 Tasks

Monitoring:

- AP/client health dashboards

- Channel utilization and interference

- Rogue AP detection & alerts

Optimization:

- Dynamic RF adjustments (or manual tuning)

- Adjustments to SSID load balancing, minimum bitrate settings

Security:

- Firmware upgrades

- WPA3 rollout, segmentation enforcement

- Guest traffic isolation

Change management:

- Adding new APs or sites

- Adjusting RF or network profiles

Analytics/Reporting:

- Usage reports

- Application performance

- Client behavior/location tracking

Wi-Fi Survey by Project Phase

| Project Phase | Survey Type | Purpose |
| --- | --- | --- |
| Plan & Design | Wi-Fi Predictive Design | Design RF coverage before deployment using building floor plans and RF modeling tools. |
| Pre-Deployment | Wi-Fi Pre-deployment Survey | Validate predictive designs onsite using test APs to measure real-world attenuation and interference. |
| Deployment Validation | Wi-Fi Deployment Validation Survey | Post-installation survey to confirm coverage, signal quality, and validate that the design has been implemented successfully. |
| Operations / Troubleshooting | Wi-Fi Diagnostic Survey | Performed when users report issues; it identifies RF problems, interference, misconfigurations, or performance bottlenecks. |

802.1X is the gold standard for port-based network access control in enterprise Wi-Fi

## 802.1X Authentication Flow in Wi-Fi

- Supplicant connects to AP (SSID using 802.1X)

- AP (Authenticator) forwards request to RADIUS server

- RADIUS Server checks identity & authentication policy

- If successful:

  - Port is opened

  - Role/VLAN/DACLs may be pushed

- If failed:

  - Access denied or fallback to guest VLAN

## Common EAP Types in 802.1X Wi-Fi

| EAP Type | Description | Use Case |
| --- | --- | --- |
| EAP-TLS | Cert-based mutual auth | Highly secure, for corporate devices |
| PEAP-MSCHAPv2 | Username/password in tunnel | Easy to deploy, common with AD |
| EAP-TTLS | Tunnel with varied auth | Flexible for mixed environments |
| EAP-FAST | Cisco proprietary | Legacy Cisco environments |

## Core Components of 802.1X in Wi-Fi

### 1. Supplicant (Client Device)

- The software or device requesting access to the network.

- Usually built into OS (Windows, macOS, iOS, Android, Linux).

- Examples:

  - Laptops, phones, tablets

  - IoT devices (may require certificate-based auth or MAC bypass)

- Can use:

  - EAP-TLS (cert-based)

  - PEAP/MSCHAPv2 (username/password)

### 2. Authenticator (Access Point or Controller)

- Wireless access point (AP) or wireless controller that acts as the middleman between the supplicant and the authentication server.

- Controls port access (open or restrict) based on authentication result.

- Common examples:

  - Cisco Catalyst/Meraki, Aruba, Juniper Mist APs

### 3. Authentication Server (RADIUS Server)

- Validates the credentials or certificates received via the authenticator.

- Often integrates with identity stores like:

  - Microsoft Active Directory

  - LDAP

  - Azure AD (via extensions)

- Common RADIUS solutions:

  - Cisco ISE

  - Aruba ClearPass

  - FreeRADIUS

  - Windows NPS

## Supporting Components

### 4. Certificate Authority (CA)

- Required for EAP-TLS, PEAP, or TTLS to issue and manage digital certificates.

- Can be:

  - Microsoft AD Certificate Services

  - Public CAs (for BYOD use cases)

  - Internal PKI

### 5. Identity Store

- Backend user directory (e.g., AD, LDAP) used by the RADIUS server to validate user/device credentials.

### 6. Policy Engine (optional but common)

- Provides role-based access, VLAN assignments, downloadable ACLs.

- Can apply differentiated access based on user group, device posture, time of day, etc.

Examples: Cisco ISE or Aruba ClearPass

Tools

Wi-Fi SSID Overhead Calculator: revolution wifi

ASSUMPTIONS:

802.11b Long Preamble used for 1 Mbps; Short Preamble used for 2, 5.5, 11 Mbps

802.11g short slot time is assumed, with no 802.11b clients within range

WMM is enabled and beacons are transmitted using Best Effort AC

| VARIABLES: |  |
| --- | --- |
| Beacon Data Rate (Mbps) | 802.11a 12 Mbps |
| Beacon Frame Size (Bytes) | 330 |
| Beacon Interval (ms) | 102.4 |

Amount of Overhead:

| 0-10% Low | 10-20% Medium | 20-50% High | >50% Very High |
| --- | --- | --- | --- |

| Number of APs on Channel* | Number of SSIDs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |

## Standards

Channel Planning

https://www.ekahau.com/blog/channel-planning-best-practices-for-better-wi-fi/

http://revolutionwifi.blogspot.com/2013/03/80211ac-channel-planning.html

wireless-enterprise-authentication-802-1x

WPA Authentication Protocols – Overview & Comparison

| Protocol | Authentication Type | Security Level | Typical Use Case |
| --- | --- | --- | --- |
| WPA2-PSK | Shared Password | Moderate | Small businesses, guest Wi-Fi |
| WPA3-PSK (SAE) | Shared Password | Stronger | Newer guest networks |
| WPA2-Enterprise | 802.1X + RADIUS | Strong | Standard for enterprise |
| WPA3-Enterprise | 802.1X + RADIUS + 192-bit crypto | Highest | High security/government |

| Mode | Uses 802.1X? | How Authentication Works |
| --- | --- | --- |
| WPA2-Enterprise | Yes | Auth via 802.1X + RADIUS + EAP (PEAP, EAP-TLS, etc.) |
| WPA3-Enterprise | Yes | Same as above but with enhanced encryption (192-bit) |
| WPA2-PSK | No | Shared password (no identity, no 802.1X) |
| WPA3-PSK (SAE) | No | Shared password with improved security (still no 802.1X) |

What 802.1X brings to the table:

- Per-user/device authentication (identity-aware access)

- Integration with RADIUS servers (e.g., Cisco ISE, Cloud RADIUS)

- Enables dynamic VLAN assignment, NAC, posture checks

- Supports EAP types (e.g., PEAP, EAP-TLS for cert-based auth)

So in short:
Enterprise = 802.1X + RADIUS + identity-based access
PSK = just a password, no identity layer

WPA authentication protocols in different modes:

- WPA2-Enterprise

- WPA3-Enterprise

- WPA2-PSK

- WPA3-PSK

Enterprise setting, customers must establish a secure connection with an authentication server before entering their credentials.

### What Enterprises Typically Do

| SSID Type | Recommended Protocol | Why |
| --- | --- | --- |
| Corporate Devices | WPA2-Enterprise / WPA3-Enterprise | Identity-based, cert-based, VLAN support |
| BYOD/Contractors | WPA2-Enterprise + SCEP | PKI-based onboarding via Intune or Jamf |
| Guest Wi-Fi | WPA3-PSK (or open + captive portal) | Secure but frictionless access |
| IoT Devices | WPA2-PSK with MAB fallback | Simplified provisioning, isolated VLAN |

### Summary

- Use WPA2/WPA3-Enterprise for any internal/corporate SSID.

- Use PSK (preferably WPA3-PSK) for guest or IoT where 802.1X is not viable.

- Futureproof with WPA3 only if your device fleet supports it.

## Meraki Specifics

Meraki Campus Design

Simple Onboarding and Access

• Order can have a single or multiple device/license

• Power up devices and they pull config from dashboard

Role Base Access (RBAC) with Meraki Dashboard

Using tags for additional Flexibility

Integration at scale with APIs

http://developer.cisco.com/meraki/

Wireless Firmware Upgrades

Intelligent firmware rollout that constantly monitors firmware globally

### Meraki Tags: Overview

There are two main types of tags:

| Tag Type | Applies To | Purpose |
| --- | --- | --- |
| Network Tags | Entire network org/site | Organize and filter networks for templates, firmware, settings |
| Device Tags | Individual APs, switches, MX | Group devices within networks for policy, SSID, VLAN or firewall overrides |

### Network Tags

- Defined in: Meraki Dashboard > Organization > Overview

- Used for: Binding configuration templates to groups of networks (e.g., HQ, Branch, Retail)

- Use Cases:

  - Assign specific firmware versions

  - Apply common template to tagged branches (e.g., all retail-store-*)

  - Filter networks in bulk actions or dashboard views

### Device Tags

- Defined in: Device details or inventory

- Used for:

  - Per-device policy assignment, especially for wireless SSIDs and firewall rules

  - Overriding settings within a template-bound network

- Use Cases:

  - Only advertise a specific SSID on tagged APs (e.g., only lobby-ap)

  - Assign VLAN or RF profile to tagged APs

  - Enable local breakouts only on certain devices (e.g., sd-wan-primary)

### Day 0 / Day 1 / Day 2 Use Cases

| Phase | Use of Tags |
| --- | --- |
| Day 0 (Planning) | - Define network tags for branches/sites types (e.g., store, warehouse, hq) - Decide tagging schema and automation triggers - Design template hierarchy based on network tags |
| Day 1 (Deployment) | - Apply device tags to APs or switches for specific SSIDs, RF profiles, VLANs - Use network tags to bind to templates - Set RADIUS VLAN assignment overrides using tags |
| Day 2 (Operations) | - Automate firmware upgrades or configuration updates by tag - Bulk assign or audit device configurations - Filter alerts, events, and logs based on tags - Trigger Ansible/automation workflows per tag group |

### Tags + Ansible

With Meraki’s API + Ansible modules, tags help:

- Dynamically apply configs (SSID, VLANs, templates)

- Roll out changes to specific groups (e.g., ap-north-zone)

- Perform config drift detection per tag group

Roaming

Exec Summary

| Option | Description | Pros | Cons |
| --- | --- | --- | --- |
| 1. Stretched VLAN via L2 Trunking | Extend same VLAN across buildings using physical L2 trunk links | Simple design; native Meraki support | STP/scalability limits; requires contiguous switching domain |
| 2. Meraki Layer 3 Roaming with Central NAT | Meraki feature that keeps client IP via tunnel to anchor AP | Seamless roaming: no L2 extension needed | Slight overhead; configuration per SSID |
| 3. VXLAN Overlay (Hybrid) | Use Catalyst VXLAN/SD-Access fabric to stretch VLANs over IP | Scalable; modern campus core integration | Requires Catalyst core; not native to Meraki MS switches |
| 4. RADIUS + Dynamic VLAN Assignment | Central auth assigns same VLAN at all sites (requires pre-provisioned VLAN) | Policy-driven; identity aware | VLAN must exist in all sites; does not eliminate L2 constraint |

Comparing Option 2: Meraki L3 Roaming with Central NAT vs. Option 4: Dynamic VLAN via RADIUS

| Factor | Option 2: Meraki L3 Roaming w/ Central NAT | Option 4: Dynamic VLAN Assignment via RADIUS |
| --- | --- | --- |
| Roaming Experience | Seamless — user keeps same IP across APs, even across subnets | Not seamless across buildings unless VLAN exists in both areas |
| Authentication Flow | No re-auth needed when roaming — session is preserved via tunnel anchor | Re-authentication may occur when moving between VLANs/sites |
| IP Address Preservation | ✅ Yes — IP address is centrally anchored, no DHCP renewal | ❌ No — if VLAN changes across buildings, new DHCP lease is issued |
| Application Impact | Minimal — apps like VoIP, VPN, video conferencing remains stable | Risk of session drop or latency as IP stack resets during re-auth |
| User Disruption | Near-zero — users roam invisibly across infrastructure | Moderate — short outage or disconnect may be observed during reassociation |
| Configuration Complexity | Moderate — Meraki-specific config per SSID | Moderate — requires consistent RADIUS policies + pre-defined VLANs per site |
| Operational Overhead | Low once configured — no VLAN sprawl required | Higher — requires VLAN planning and provisioning across all locations |
| Ideal For | Seamless experience across multi-building campuses | Policy-driven access with centralized control (e.g., role-based VLANs) |

Summary Recommendations:

- Option 2 (Meraki L3 Roaming with Central NAT)
✅ Best for user experience in campus environments — users roam without dropping IP or session.
Ideal for real-time apps (VoIP, Zoom, Citrix, Teams).

- Option 4 (Dynamic VLAN via RADIUS)
⚠️ More suitable for policy control, but less ideal for roaming continuity.
Use where identity-based segmentation is more important than session persistence.

## Client-Side Roaming Optimization by OS

| OS / Platform | Optimization | Layer 2 or Layer 3 Benefit | Admin Action Needed |
| --- | --- | --- | --- |
| Windows 10/11 | - Enable Fast Roaming (802.11r) support via Group Policy - Enable Aggressive roaming (Roaming Aggressiveness = High) - Configure WLAN profiles via Intune/GPO - Use native WLAN AutoConfig behaviors properly | L2 & L3 (Fast reassociation and PMK caching) | Yes – via GPO, Intune, or manual regedit |
| macOS (Monterey+) | - macOS supports 802.11r, 802.11k, 802.11v natively - Use Jamf or MDM to enforce known networks - Disable power save modes aggressively in roaming-prone environments | L2 roaming (best case with Meraki Fast Roaming) | Moderate – Jamf Pro profile enforcement |
| iOS/iPadOS | - Native support for 802.11r/k/v - MDM-based Wi-Fi config to optimize SSID selection - Background scanning on roaming candidates | L2 roaming | No extra config needed beyond MDM |
| Android (11+) | - Varies by OEM; enforce roaming parameters using Android Enterprise - Some support for 802.11k/r/v but inconsistently - Disable Wi-Fi sleep/power save on critical apps | L2 roaming (if supported) | OEM/MDM-dependent |
| Windows (legacy) | - Roaming delay due to lack of 802.11r/OKC - Use driver updates from Intel/Qualcomm - Avoid GPOs that restrict wireless profile flexibility | May degrade roaming unless patched | Requires manual tuning or OS upgrade |

## General Client-Side Best Practices

- Use certificate-based authentication (EAP-TLS): avoids interactive prompts during reassociation.

- Limit SSID count per AP: fewer SSIDs = faster scans = quicker roaming decisions.

- Driver Updates: ensure Wi-Fi adapter drivers (Intel, Broadcom, etc.) are up to date — many roaming fixes are vendor-specific.

- Avoid power-save modes on critical roaming clients (VoIP handsets, thin clients).

- Fast Transition (FT/802.11r) should be explicitly supported in the SSID — most enterprise clients benefit greatly.

## For Meraki Networks Specifically:

Meraki supports:

- 802.11r (Fast BSS Transition)

- 802.11k (Neighbor reporting)

- 802.11v (BSS Transition Management)

Clients must support these as well for them to function — so platform tuning + Meraki config go hand-in-hand.

## BYOD Self-Provisioning with Cisco ISE (WPA2/WPA3-Enterprise)

SSID: BYOD-WiFi
Security Type: WPA2-Enterprise or WPA3-Enterprise
Use Case: Staff/contractor personal devices that need secure corporate access but are not fully managed.

### Trust Enforcement Logic

| Stage | Function | Enforced By | Notes |
| --- | --- | --- | --- |
| Connect to Onboarding SSID | User connects to open or pre-auth SSID (BYOD-Onboard) | Meraki AP | Triggers redirection |
| Device Identity Check | ISE portal identifies device as unmanaged | Cisco ISE | MAC and browser fingerprinting |
| Self-Service Portal | User logs in with corporate SSO | Cisco ISE Portal | Enforced via SAML or LDAP |
| Certificate Provision | ISE issues personal certificate via SCEP | ISE + ISE Internal CA or External CA | Cert includes user/device identity |
| Network Switch | ISE dynamically redirects to BYOD-WiFi SSID | ISE + Meraki RADIUS change-of-authorization (CoA) | Enforces WPA2/3-Enterprise with EAP-TLS |
| Role-Based Access | Assign VLAN/ACL based on identity group | ISE + Meraki Group Policy | Personal devices restricted from privileged apps |

### Optional Enhancements:

- Posture assessment via ISE Agent or browser-based scan (e.g., AV installed, updates current).

- MDM enrollment redirect if corporate ownership changes.

- Guest fallback: If identity fails, redirect to Guest-WiFi.

## MDM Policy Enforcement Logic – EAP-TLS Workflow (Zero Trust Aligned)

Objective: Ensure only MDM-enrolled corporate devices with valid certificates can join the enterprise SSID.

### Trust Policy Flow:

| Stage | Function | Enforced By | Notes |
| --- | --- | --- | --- |
| Device Enrollment | Device registers with MDM (e.g., Intune, JAMF) | MDM Platform | Tagged as corporate asset |
| Certificate Issuance | Unique client certificate issued (EAP-TLS) | SCEP/NDES or MDM CA | Includes device identifier (UDID, hostname) |
| Wi-Fi Join Attempt | Device attempts WPA3-Enterprise connection | Meraki AP | Triggers 802.1X |
| Identity Validation | Certificate checked (CN, SAN, expiry) | Cisco ISE / RADIUS | Policy matched via certificate attributes |
| Posture Check (optional) | Validate compliance (OS version, antivirus, disk encryption) | Cisco ISE + MDM API | Can assign different VLANs or deny access |
| Network Access | VLAN assigned + group policy enforced | Meraki + RADIUS | Access granted only if policy matches |

### Zero Trust Design Principles Supported:

- Strong Identity Assurance: Device + User identity bound to cert

- Least Privilege: Dynamic VLANs and ACLs per posture

- Continuous Evaluation: ISE can revalidate posture during session

## WPA3-Personal Guest Wi-Fi – Zero Trust Guest Access Logic

SSID: Guest-WiFi
Security Type: WPA3-Personal (SAE)
Use Case: Short-term guest users with simplified access and basic isolation.

### Trust Enforcement Logic

| Stage | Function | Enforced By | Notes |
| --- | --- | --- | --- |
| Broadcast SSID | Guest-WiFi with WPA3-Personal (SAE) | Meraki AP | Transition mode optional for legacy support |
| Pre-shared Key Entry | User inputs dynamic or rotating PSK | User Device | Optional PSK rotation via automation |
| DHCP Assignment | Guest IP assigned from DMZ/guest VLAN | Meraki or upstream DHCP | VLAN segmentation ensures traffic isolation |
| Content & URL Filtering | Limit access to corporate or sensitive assets | Meraki L7 firewall rules | Can integrate with Umbrella or firewall content filters |
| Session Tracking | Optional splash page agreement or click-through | Meraki Captive Portal | No identity, but legal/log compliance enforced |

### Optional Enhancements:

- Use sponsored guest registration portals (Meraki or ISE) for known guests.

- Integrate with Umbrella DNS Filtering for outbound DNS control.

- Rotate PSKs on schedule via Meraki API.