Wireless Enterprise Standard

# Status: Draft

Wireless Day0, 1 and 2 Tasks

Monitoring:

- AP/client health dashboards

- Channel utilization and interference

- Rogue AP detection & alerts

Optimization:

- Dynamic RF adjustments (or manual tuning)

- Adjustments to SSID load balancing, minimum bitrate settings

Security:

- Firmware upgrades

- WPA3 rollout, segmentation enforcement

- Guest traffic isolation

Change management:

- Adding new APs or sites

- Adjusting RF or network profiles

Analytics/Reporting:

- Usage reports

- Application performance

- Client behavior/location tracking

Wi-Fi Survey by Project Phase

| Project Phase | Survey Type | Purpose |
| --- | --- | --- |
| Plan & Design | Wi-Fi Predictive Design | Design RF coverage before deployment using building floor plans and RF modeling tools. |
| Pre-Deployment | Wi-Fi Pre-deployment Survey | Validate predictive designs onsite using test APs to measure real-world attenuation and interference. |
| Deployment Validation | Wi-Fi Deployment Validation Survey | Post-installation survey to confirm coverage, signal quality, and validate that the design has been implemented successfully. |
| Operations / Troubleshooting | Wi-Fi Diagnostic Survey | Performed when users report issues; it identifies RF problems, interference, misconfigurations, or performance bottlenecks. |

802.1X is the gold standard for port-based network access control in enterprise Wi-Fi

## 802.1X Authentication Flow in Wi-Fi

- Supplicant connects to AP (SSID using 802.1X)

- AP (Authenticator) forwards request to RADIUS server

- RADIUS Server checks identity & authentication policy

- If successful:

  - Port is opened

  - Role/VLAN/DACLs may be pushed

- If failed:

  - Access denied or fallback to guest VLAN

## Common EAP Types in 802.1X Wi-Fi

| EAP Type | Description | Use Case |
| --- | --- | --- |
| EAP-TLS | Cert-based mutual auth | Highly secure, for corporate devices |
| PEAP-MSCHAPv2 | Username/password in tunnel | Easy to deploy, common with AD |
| EAP-TTLS | Tunnel with varied auth | Flexible for mixed environments |
| EAP-FAST | Cisco proprietary | Legacy Cisco environments |

## Core Components of 802.1X in Wi-Fi

### 1. Supplicant (Client Device)

- The software or device requesting access to the network.

- Usually built into OS (Windows, macOS, iOS, Android, Linux).

- Examples:

  - Laptops, phones, tablets

  - IoT devices (may require certificate-based auth or MAC bypass)

- Can use:

  - EAP-TLS (cert-based)

  - PEAP/MSCHAPv2 (username/password)

### 2. Authenticator (Access Point or Controller)

- Wireless access point (AP) or wireless controller that acts as the middleman between the supplicant and the authentication server.

- Controls port access (open or restrict) based on authentication result.

- Common examples:

  - Cisco Catalyst/Meraki, Aruba, Juniper Mist APs

### 3. Authentication Server (RADIUS Server)

- Validates the credentials or certificates received via the authenticator.

- Often integrates with identity stores like:

  - Microsoft Active Directory

  - LDAP

  - Azure AD (via extensions)

- Common RADIUS solutions:

  - Cisco ISE

  - Aruba ClearPass

  - FreeRADIUS

  - Windows NPS

## Supporting Components

### 4. Certificate Authority (CA)

- Required for EAP-TLS, PEAP, or TTLS to issue and manage digital certificates.

- Can be:

  - Microsoft AD Certificate Services

  - Public CAs (for BYOD use cases)

  - Internal PKI

### 5. Identity Store

- Backend user directory (e.g., AD, LDAP) used by the RADIUS server to validate user/device credentials.

### 6. Policy Engine (optional but common)

- Provides role-based access, VLAN assignments, downloadable ACLs.

- Can apply differentiated access based on user group, device posture, time of day, etc.

Examples: Cisco ISE or Aruba ClearPass

Tools

Wi-Fi SSID Overhead Calculator: revolution wifi

ASSUMPTIONS:

802.11b Long Preamble used for 1 Mbps; Short Preamble used for 2, 5.5, 11 Mbps

802.11g short slot time is assumed, with no 802.11b clients within range

WMM is enabled and beacons are transmitted using Best Effort AC

| VARIABLES: |  |
| --- | --- |
| Beacon Data Rate (Mbps) | 802.11a 12 Mbps |
| Beacon Frame Size (Bytes) | 330 |
| Beacon Interval (ms) | 102.4 |

Amount of Overhead:

| 0-10% Low | 10-20% Medium | 20-50% High | >50% Very High |
| --- | --- | --- | --- |

| Number of APs on Channel* | Number of SSIDs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |

## Standards

Channel Planning

https://www.ekahau.com/blog/channel-planning-best-practices-for-better-wi-fi/

http://revolutionwifi.blogspot.com/2013/03/80211ac-channel-planning.html

wireless-enterprise-authentication-802-1x

WPA Authentication Protocols – Overview & Comparison

| Protocol | Authentication Type | Security Level | Typical Use Case |
| --- | --- | --- | --- |
| WPA2-PSK | Shared Password | Moderate | Small businesses, guest Wi-Fi |
| WPA3-PSK (SAE) | Shared Password | Stronger | Newer guest networks |
| WPA2-Enterprise | 802.1X + RADIUS | Strong | Standard for enterprise |
| WPA3-Enterprise | 802.1X + RADIUS + 192-bit crypto | Highest | High security/government |

| Mode | Uses 802.1X? | How Authentication Works |
| --- | --- | --- |
| WPA2-Enterprise | Yes | Auth via 802.1X + RADIUS + EAP (PEAP, EAP-TLS, etc.) |
| WPA3-Enterprise | Yes | Same as above but with enhanced encryption (192-bit) |
| WPA2-PSK | No | Shared password (no identity, no 802.1X) |
| WPA3-PSK (SAE) | No | Shared password with improved security (still no 802.1X) |

What 802.1X brings to the table:

- Per-user/device authentication (identity-aware access)

- Integration with RADIUS servers (e.g., Cisco ISE, Cloud RADIUS)

- Enables dynamic VLAN assignment, NAC, posture checks

- Supports EAP types (e.g., PEAP, EAP-TLS for cert-based auth)

So in short:
Enterprise = 802.1X + RADIUS + identity-based access
PSK = just a password, no identity layer

WPA authentication protocols in different modes:

- WPA2-Enterprise

- WPA3-Enterprise

- WPA2-PSK

- WPA3-PSK

Enterprise setting, customers must establish a secure connection with an authentication server before entering their credentials.

### What Enterprises Typically Do

| SSID Type | Recommended Protocol | Why |
| --- | --- | --- |
| Corporate Devices | WPA2-Enterprise / WPA3-Enterprise | Identity-based, cert-based, VLAN support |
| BYOD/Contractors | WPA2-Enterprise + SCEP | PKI-based onboarding via Intune or Jamf |
| Guest Wi-Fi | WPA3-PSK (or open + captive portal) | Secure but frictionless access |
| IoT Devices | WPA2-PSK with MAB fallback | Simplified provisioning, isolated VLAN |

### Summary

- Use WPA2/WPA3-Enterprise for any internal/corporate SSID.

- Use PSK (preferably WPA3-PSK) for guest or IoT where 802.1X is not viable.

- Futureproof with WPA3 only if your device fleet supports it.